JRM Chemical, Inc.

MSDS

MATERIAL SAFETY DATA SHEET              Page 1 of  2

PRODUCT 	 TRADEMARK     H-100 ENVIRONMENTAL SPILL ENCAPSULANT

IDENTIFICATION

SYNONYMS                          None

CHEMICAL FAMILY  	     Homopolymer

FORMULATION                   Proprietary

FIRE AND EXPLOSION HAZARDS

FLASH POINT                      N/A

AUTO IGNITION TEM        440C

FLAMMABLE LIMITS

IN AIR                                   N/A

HEALTH HAZARDS

INHALATION                     Dust may be irritating to respiratory tract

SKIN                                     Dust may cause slight skin irritation if skin contact is repeated or

prolonged use.

EYES                                    Dust if contacts the eye may be irritating.

INGESTION                         If ingestion occurs, may be irritating to GI tract.

CHRONIC TOXICITY        No effects from chronic exposure are known.

TOXICOLOGY                    No-NTP

CARCINOGENICITY         No-IARC

FIRST AID MEASURES

EYES                                   Immediately flush with water for 15 minutes.  Seek medical attention if

irritation occurs.

SKIN                                   Wash thoroughly with soap and water.  Seek medical attention if irritation

occurs.

INHALATION                    No harmful effects are anticipated with low levels of dust.  If problems

develop remove individual from work area and seek medical attention.

INGESTION                       No harmful effects are anticipated.  Seek medical attention if irritation persists.

PRECAUTIONS AND PROCEDURES

HAZARDOUS COMBUSTION

PRODUCTS                       Irritating and/or toxic gases due to decomposition of the product may be

generated during a fire.

SPECIAL FIRE FIGHT

PRECAUTIONS                In the event of fire, wear full protective clothing and NIOSH approved self

contained breathing apparatus.

FIRE EXTINGUISHING

AGENTS RECOMMENDED     Water spray, foam, carbon dioxide, dry chemicals.

STORAGE AND HANDLING   Store in a cool dry place and protect from moisture contamination.

Product becomes slippery when wet and creates hazard on walking

surfaces.

Use good housekeeping practices during storage, transfer and handling

to avoid excessive dust accumulation.

VENTILATION                          Provide exhaust ventilation where needed.

MATERIAL SAFETY DATA SHEET                              Page 2 of 2

PERSONAL PROTECTIVE EQUIPMENT

For conditions where exposure to dust and fumes is great, use a

NOISH approved respirator for dust, mists and fumes.  For eyes

and face, use safety glasses with side shields.  Wear clean body

covering and gloves impervious to dust or vapor to minimize

skin contact.

PHYSICAL DATA

MATERIAL                              Solid

APPEARANCE                        White powder

BOILING POINT                      N/A

MELTING POINT                    N/A

SPECIFIC GRAVITY              .55 - .65

VAPOR DENSITY                   N/A

SOLUBILITY IN H20              Insoluble

%VOLATILES                         N/A

EVAPORATION RATE          N/A

REACTIVITY DATA

STABILITY                             Stable

INCOMPATIBILITY              Strong oxidizing agents

HAZARDOUS

POLYMERICATION             Will not occur

CONDITIONS TO AVOID    Open flame

ENVIRONMENTAL INFORMATION

SPILL                                          Clean spill in manner that does not disperse dust in the air.

Extinguish all ignition sources.  Product is very slippery when

wet and causes hazards.

RECOMMEND DISPOSAL      Dispose of solid waste observing all local, state and federal

regulations.

MISCELLANEOUS

HMIS:                                        HEALTH-1   FIRE-1  REACTIVITY-1 PROTECTION-X

DOT NAME                              Synthetic Resin, power

DOT HAZARD CLASS           N/A

TSCA STATUS                        All ingredients are listed on TSCA Inventory.

This information is given without any warranty or representation.  We do not assume any legal responsibility for same.  It is offered solely for your consideration, investigation and verification.  Before using any product, read its label.

REVISION 6/96

JRM Chemical, Inc.

4881 NEO Parkway

Cleveland, OH  44128

Phone:  216-475-8488

Fax:	  216-475-6517